Step 1: Open a terminal and type the following command:

$ dmesg | grep tty    # This will give USB port in which GPS is connected

$ lsusb

Look for a line that mentions something like "Silicon Labs CP210x USB to UART Bridge".

Step2:  sudo cat /dev/ttyUSB0 > gps_output1.txt

(Please note USB can be USB0, USB1 etc. depend on which port gps module is connected)

This will save the output like

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GLGSA,A,1,,,,,,,,,,,,,,,*02

$GNRMC,073936.089,V,,,,,0.53,162.90,041024,,,N*53

$GNVTG,162.90,T,,M,0.53,N,0.97,K,N*28

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GLGSA,A,1,,,,,,,,,,,,,,,*02

$GNRMC,073936.089,V,,,,,0.53,162.90,041024,,,N*53

$GNVTG,162.90,T,,M,0.53,N,0.97,K,N*28

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

If you want only GPGSA (which is required in our case filter out other, in that case step 2 is  not required)

Step 3:  cat /dev/ttyUSB1 | grep '\$GPGSA' > gpgsa_output.txt

If this is not work then

$ sudo cat /dev/ttyUSB1 | grep '\$GPGSA' > gpgsa_output.txt

$ sudo cat /dev/ttyUSB0 | grep '\$GNRMC’ > gnrmc_output.txt

sanjita@sanjita-Precision-3660:~$ cat /dev/ttyUSB0 | grep '\$GNRMC' > gnrmc_output111024.txt

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

$GPGSA,A,1,,,,,,,,,,,,,,,*1E

Step 4: to stop the  recording

Type command

Press ctrl + C button

How to run GPS

Step 1 Start GPS

Go to directory where file start_gps.sh is present in our case

sanjita@sanjita-Precision-3660:~/GPS$ ./start_gps.sh

Step 2 Stop GPS recording

click on + button and open new terminal on the terminal where gps is running.

Run: ps aux | grep gps_script.py

sanjita@sanjita-Precision-3660:~/GPS$ ps aux | grep gps_script.py

this will give following output

sanjita     4652 96.0  0.0  18908 10212 pts/2    R+   18:54   6:06 python3 /home/sanjita/GPS/gps_script.py

sanjita     4842  0.0  0.0   9044   712 pts/3    S+   19:01   0:00 grep --color=auto gps_script.py

Now type

kill <PID>

kill 4652   (Please note this 4652 will change every time so we have keep the PID which is coming first line of running ps aux | grep gps_script.py in this case 4652)

sanjita@sanjita-Precision-3660:~/GPS$ kill 4652

(Please note 4652 is not fixed)

This will terminate the GPS recording